POAP IMPLEMENTATION GUIDE

Power On Auto Provisioning

Red Hat Linux  +  Cisco Nexus Management Switch  +  Nexus 3K

| Document Information |  |
| --- | --- |
| Document Version | 1.1 — Red Hat Edition |
| Date | March 2026 |
| OS | Red Hat Enterprise Linux (RHEL) 8/9 |
| Package Manager | dnf (replaces apt) |
| Firewall Tool | firewalld (replaces ufw) |
| Network Config | nmcli / nmtui (replaces netplan) |
| DHCP Server | dhcp-server (replaces isc-dhcp-server) |
| TFTP Server | tftp-server (replaces tftpd-hpa) |
| Target Switch | Cisco Nexus 3K — NX-OS 7.0.3.I7.6 |

⚠️ KEY DIFFERENCES — Red Hat vs Ubuntu apt → dnf ufw → firewalld netplan → nmcli / nmtui isc-dhcp-server → dhcp-server tftpd-hpa → tftp-server (via xinetd or systemd) /var/log/syslog → journalctl -u dhcpd / journalctl -u tftp

# 1.  Network Topology

POAP TOPOLOGY ═══════════════════════════════════════════════════════════ [Red Hat Linux Server 192.168.1.100] ├── DHCP Server (dhcp-server) └── TFTP Server (tftp-server) ¦ ¦ Eth1/1 — access VLAN 100 ¦ [Management Switch — Cisco Nexus] SVI VLAN 100: 192.168.1.1/24 ¦ ¦ Eth1/2 — access VLAN 100 ¦ [Nexus 3K — Target Switch] mgmt0: 192.168.1.10/24 (via DHCP) ═══════════════════════════════════════════════════════════

# 2.  IP Addressing Plan

| IP Addressing Plan |  |
| --- | --- |
| Red Hat Linux Server | 192.168.1.100 / 24 |
| Management Switch SVI | 192.168.1.1 / 24 |
| Nexus 3K mgmt0 | 192.168.1.10 / 24 (via DHCP) |
| DHCP Pool | 192.168.1.10 — 192.168.1.50 |
| POAP Management VLAN | VLAN 100 |
| TFTP Files Location | /var/lib/tftpboot/ on Red Hat Server |

# 3.  POAP Workflow — Step by Step

1. Nexus 3K boots — no startup-config — POAP starts 2. Switch sends DHCP Discover out of mgmt0 3. Management switch forwards to Red Hat Server via VLAN 100 4. dhcp-server responds: IP: 192.168.1.10 ¦ Option 66: 192.168.1.100 ¦ Option 67: poap_script.py 5. Switch downloads poap_script.py via TFTP 6. Script executes on NX-OS Python 2.7 7. Script downloads switch_config.cfg via TFTP 8. Script applies config to startup-config 9. Switch reloads — comes up with full configuration ✅

# 4.  Red Hat Linux Server Configuration

STEP 1 — Update System Run as root or with sudo

sudo dnf update -y

STEP 2 — Set Static IP Using nmcli — Red Hat network manager

Find your interface name:

ip a

nmcli device status

📝  Interface is usually eth0, ens33, enp0s3 — note it down

Set static IP using nmcli:

# Replace 'eth0' with your actual interface name

sudo nmcli con mod eth0 ipv4.addresses 192.168.1.100/24

sudo nmcli con mod eth0 ipv4.gateway 192.168.1.1

sudo nmcli con mod eth0 ipv4.dns '8.8.8.8 8.8.4.4'

sudo nmcli con mod eth0 ipv4.method manual

sudo nmcli con up eth0

Alternative — use nmtui (graphical TUI):

sudo nmtui

📝  Select 'Edit a connection' → choose interface → set IPv4 to Manual → enter 192.168.1.100/24

Verify:

ip a show eth0

📝  Must show: inet 192.168.1.100/24

⚠️  Red Hat does NOT use netplan — nmcli/nmtui replaces it completely

| ❌ Ubuntu (NOT used) sudo nano /etc/netplan/00-installer-config.yaml sudo netplan apply | ✅ Red Hat (USE THIS) sudo nmcli con mod eth0 ipv4.addresses 192.168.1.100/24 sudo nmcli con up eth0 |
| --- | --- |

STEP 3 — Install & Configure DHCP Server dhcp-server — Red Hat package name

Install:

sudo dnf install dhcp-server -y

Configure DHCP with POAP options:

sudo nano /etc/dhcp/dhcpd.conf

# Delete everything and paste this:

default-lease-time 3600;

max-lease-time 7200;

authoritative;

subnet 192.168.1.0 netmask 255.255.255.0 {

range 192.168.1.10 192.168.1.50;

option routers 192.168.1.100;

option subnet-mask 255.255.255.0;

option tftp-server-name "192.168.1.100";

# Option 66 — TFTP Server IP

option bootfile-name "poap_script.py";

# Option 67 — POAP Script filename

next-server 192.168.1.100;

}

Set interface in sysconfig:

sudo nano /etc/sysconfig/dhcpd

# Add or edit this line:

DHCPDARGS=eth0

📝  Replace eth0 with your actual interface name

Start and enable at boot:

sudo systemctl start dhcpd

sudo systemctl enable dhcpd

sudo systemctl status dhcpd

⚠️  Status must show: active (running)

| ❌ Ubuntu (NOT used) sudo apt install isc-dhcp-server -y INTERFACESv4="eth0" in /etc/default/isc-dhcp-server sudo systemctl start isc-dhcp-server | ✅ Red Hat (USE THIS) sudo dnf install dhcp-server -y DHCPDARGS=eth0 in /etc/sysconfig/dhcpd sudo systemctl start dhcpd |
| --- | --- |

STEP 4 — Install & Configure TFTP Server tftp-server — served via systemd socket on Red Hat

Install:

sudo dnf install tftp-server -y

Enable the TFTP socket (systemd based on RHEL 8/9):

sudo systemctl enable --now tftp.socket

sudo systemctl start tftp.socket

sudo systemctl status tftp.socket

TFTP root directory on Red Hat:

ls /var/lib/tftpboot/

📝  Red Hat TFTP root is /var/lib/tftpboot/ — NOT /srv/tftp/ like Ubuntu

Create files directory and set permissions:

sudo mkdir -p /var/lib/tftpboot

sudo chmod 755 /var/lib/tftpboot

sudo chown -R nobody:nobody /var/lib/tftpboot

⚠️  Red Hat uses /var/lib/tftpboot/ as TFTP root — all files go here

| ❌ Ubuntu (NOT used) sudo apt install tftpd-hpa -y TFTP_DIRECTORY="/srv/tftp" sudo systemctl start tftpd-hpa | ✅ Red Hat (USE THIS) sudo dnf install tftp-server -y TFTP root = /var/lib/tftpboot/ sudo systemctl enable --now tftp.socket |
| --- | --- |

STEP 5 — Configure Firewall firewalld — Red Hat firewall — replaces ufw

Allow DHCP and TFTP services:

sudo firewall-cmd --permanent --add-service=dhcp

sudo firewall-cmd --permanent --add-service=tftp

sudo firewall-cmd --permanent --add-port=69/udp

sudo firewall-cmd --permanent --add-port=67/udp

sudo firewall-cmd --permanent --add-port=68/udp

sudo firewall-cmd --permanent --add-service=ssh

sudo firewall-cmd --reload

Verify firewall rules:

sudo firewall-cmd --list-all

✅  Must show dhcp, tftp, ssh in services list

| ❌ Ubuntu (NOT used) sudo ufw allow 67/udp sudo ufw allow 69/udp sudo ufw enable sudo ufw status | ✅ Red Hat (USE THIS) sudo firewall-cmd --permanent --add-service=dhcp sudo firewall-cmd --permanent --add-service=tftp sudo firewall-cmd --reload |
| --- | --- |

STEP 6 — Place POAP Files Files go in /var/lib/tftpboot/ on Red Hat

Create POAP script:

sudo nano /var/lib/tftpboot/poap_script.py

Create switch config:

sudo nano /var/lib/tftpboot/switch_config.cfg

Set correct ownership and permissions:

sudo chown nobody:nobody /var/lib/tftpboot/poap_script.py

sudo chown nobody:nobody /var/lib/tftpboot/switch_config.cfg

sudo chmod 644 /var/lib/tftpboot/poap_script.py

sudo chmod 644 /var/lib/tftpboot/switch_config.cfg

Verify files:

ls -la /var/lib/tftpboot/

📝  Must see both poap_script.py and switch_config.cfg owned by nobody

Test TFTP locally:

sudo dnf install tftp -y

cd /tmp && tftp 192.168.1.100

tftp> get poap_script.py

tftp> quit

ls -la /tmp/poap_script.py

✅  File downloaded = TFTP working

SELinux — Important on Red Hat:

⚠️  If TFTP fails even with correct permissions — SELinux may be blocking it

sudo setsebool -P tftp_home_dir on

sudo restorecon -Rv /var/lib/tftpboot/

📝  SELinux is enabled by default on Red Hat — this step is often needed

# 5.  Management Switch Configuration (Cisco Nexus)

STEP 5 — Configure Management Switch Same as Ubuntu setup — no changes needed here

5.1  Create POAP Management VLAN:

mgmt-switch# configure terminal

mgmt-switch(config)# vlan 100

mgmt-switch(config-vlan)# name POAP-MGMT

mgmt-switch(config-vlan)# exit

5.2  Find port numbers:

mgmt-switch# show cdp neighbors

mgmt-switch# show interface status

5.3  Configure port to Red Hat Server:

mgmt-switch(config)# interface Ethernet1/1

mgmt-switch(config-if)# description RedHat-POAP-Server

mgmt-switch(config-if)# switchport

mgmt-switch(config-if)# switchport mode access

mgmt-switch(config-if)# switchport access vlan 100

mgmt-switch(config-if)# no shutdown

mgmt-switch(config-if)# exit

5.4  Configure port to Nexus 3K mgmt0:

mgmt-switch(config)# interface Ethernet1/2

mgmt-switch(config-if)# description Nexus3K-mgmt0

mgmt-switch(config-if)# switchport

mgmt-switch(config-if)# switchport mode access

mgmt-switch(config-if)# switchport access vlan 100

mgmt-switch(config-if)# no shutdown

mgmt-switch(config-if)# exit

5.5  Create SVI for VLAN 100:

mgmt-switch(config)# feature interface-vlan

mgmt-switch(config)# interface vlan 100

mgmt-switch(config-if)# ip address 192.168.1.1/24

mgmt-switch(config-if)# no shutdown

mgmt-switch(config-if)# exit

5.6  Save and verify:

mgmt-switch# copy running-config startup-config

mgmt-switch# show vlan brief

mgmt-switch# ping 192.168.1.100 vrf default

# 6.  POAP Python Script (poap_script.py)

Save this file at: /var/lib/tftpboot/poap_script.py on the Red Hat Server

#!/usr/bin/env python

# POAP Script for Nexus 3K — Red Hat Server Edition

# NX-OS Python 2.7

import os, sys, signal, syslog

TFTP_SERVER = "192.168.1.100"

CONFIG_FILE = "switch_config.cfg"

LOG_FILE    = "/bootflash/poap.log"

def log(msg):

message = "POAP: " + str(msg)

try: syslog.syslog(9, message)

except: pass

try:

with open(LOG_FILE, "a") as f:

f.write(message + "\n")

except: pass

print(message)

def sigterm_handler(signum, frame):

log("SIGTERM - POAP timed out!"); sys.exit(1)

signal.signal(signal.SIGTERM, sigterm_handler)

def run_cli(cmd):

log("Running: " + cmd)

try:

import cli

return str(cli.cli(cmd))

except: pass

try:

import cisco

return str(cisco.cli(cmd))

except: pass

try:

import subprocess

return subprocess.check_output(["vsh","-c",cmd],

stderr=subprocess.STDOUT)

except Exception as e:

log("All CLI methods failed: " + str(e))

return ""

def tftp_get(filename, destination):

cmd = "copy tftp://{}/{} {} vrf management".format(

TFTP_SERVER, filename, destination)

run_cli(cmd)

if filename in str(run_cli("dir bootflash:/")):

log("Confirmed: " + filename); return True

cmd2 = "copy tftp://{}/{} {}".format(

TFTP_SERVER, filename, destination)

run_cli(cmd2)

if filename in str(run_cli("dir bootflash:/")):

log("Confirmed: " + filename); return True

return False

def apply_config(path):

run_cli("copy {} startup-config".format(path))

verify = run_cli("show startup-config")

return verify and len(str(verify)) > 10

def main():

log("=" * 50)

log("POAP Script Started")

log("TFTP: " + TFTP_SERVER)

log("=" * 50)

dest = "bootflash:/" + CONFIG_FILE

if not tftp_get(CONFIG_FILE, dest):

log("FATAL: Download failed!"); sys.exit(1)

if not apply_config(dest):

log("FATAL: Config apply failed!"); sys.exit(1)

log("POAP COMPLETED SUCCESSFULLY!")

if __name__ == "__main__": main()

# 7.  Switch Config File Rules (switch_config.cfg)

Save this file at: /var/lib/tftpboot/switch_config.cfg on the Red Hat Server

| Task / Component | Details | Status |
| --- | --- | --- |
| No 'version x.x.x' at top | Causes NX-OS version mismatch errors | ✅ REQUIRED |
| no boot poap enable | Stops POAP loop after first reboot | ✅ REQUIRED |
| Correct boot image path | Must match actual .bin on bootflash | ✅ REQUIRED |
| No unsupported features | Remove vxlangw, Vtep0, PoE if not present | ✅ REQUIRED |
| copy running-config startup-config | Must be the very last line | ✅ REQUIRED |
| Same NX-OS version as target | Prevents silent command failures | ✅ REQUIRED |

# 8.  Trigger POAP on the Nexus 3K

8.1  Erase existing config:

switch# write erase

switch# reload

When prompted:

Save changes? (y/n)    →  n

Confirm reload? (y/n)  →  y

8.2  At the POAP prompt on console:

Abort Power On Auto Provisioning [yes/skip/no]: no

📝  Type 'no' to continue with POAP provisioning

8.3  Expected console output:

POAP: Script Started

POAP: Downloading config from TFTP server...

MTFTP get operation was successful

POAP: File confirmed on bootflash

POAP: Config applied successfully!

POAP: POAP COMPLETED SUCCESSFULLY!

POAP script execution success

Reload device

# 9.  Monitoring — Red Hat Server

9.1  Watch DHCP Requests Live:

sudo journalctl -u dhcpd -f

Expected output:

DHCPDISCOVER from E4:AA:5D:87:xx:xx via eth0

DHCPOFFER on 192.168.1.10 to E4:AA:5D:87:xx:xx

DHCPREQUEST for 192.168.1.10

DHCPACK on 192.168.1.10 to E4:AA:5D:87:xx:xx

9.2  Watch TFTP Transfers Live:

sudo journalctl -u tftp.socket -f

# Or watch all system logs:

sudo journalctl -f | grep -iE 'dhcp|tftp'

Expected output:

RRQ from 192.168.1.10 filename poap_script.py

RRQ from 192.168.1.10 filename switch_config.cfg

9.3  Watch everything together:

sudo journalctl -f | grep -iE 'dhcp|tftp|poap'

9.4  Check DHCP leases:

cat /var/lib/dhcpd/dhcpd.leases

9.5  Check traffic with tcpdump:

sudo tcpdump -i eth0 port 67 or port 68   # DHCP

sudo tcpdump -i eth0 port 69               # TFTP

| ❌ Ubuntu (NOT used) sudo tail -f /var/log/syslog ¦ grep -i dhcp sudo tail -f /var/log/syslog ¦ grep -i tftp cat /var/lib/dhcp/dhcpd.leases | ✅ Red Hat (USE THIS) sudo journalctl -u dhcpd -f sudo journalctl -u tftp.socket -f cat /var/lib/dhcpd/dhcpd.leases |
| --- | --- |

# 10.  Post-POAP Verification — On Switch

| Task / Component | Details | Status |
| --- | --- | --- |
| Check hostname | show running-config ¦ include hostname | 🔧 RUN |
| Check POAP disabled | show running-config ¦ include poap | 🔧 RUN |
| Check VLANs | show vlan brief | 🔧 RUN |
| Check interfaces | show ip interface brief | 🔧 RUN |
| Check features | show running-config ¦ include feature | 🔧 RUN |
| Check startup config | show startup-config | 🔧 RUN |
| Check POAP log | show file bootflash:poap.log | 🔧 RUN |
| Check bootflash files | dir bootflash: | 🔧 RUN |

# 11.  Troubleshooting

| Task / Component | Details | Status |
| --- | --- | --- |
| Switch not getting DHCP IP | Check VLAN 100 on mgmt switch — both ports must be access VLAN 100 | 🔧 CHECK |
| DHCP not starting on Red Hat | Check config: sudo dhcpd -t -cf /etc/dhcp/dhcpd.conf | 🔧 CHECK |
| dhcpd logs not showing | Use journalctl: sudo journalctl -u dhcpd -f (not /var/log/syslog) | 🔧 CHECK |
| TFTP file not found | Verify: ls -la /var/lib/tftpboot/ — files must exist with nobody ownership | 🔧 CHECK |
| TFTP permission denied | Run: sudo restorecon -Rv /var/lib/tftpboot/ — SELinux likely blocking | 🔧 CHECK |
| SELinux blocking TFTP | Run: sudo setsebool -P tftp_home_dir on | 🔧 CHECK |
| TFTP socket not running | Run: sudo systemctl enable --now tftp.socket | 🔧 CHECK |
| Firewall blocking DHCP/TFTP | Run: sudo firewall-cmd --list-all — check dhcp and tftp in services | 🔧 CHECK |
| POAP loop after reboot | Config missing 'no boot poap enable' — add it to switch_config.cfg | 🔧 CHECK |
| Config not applying correctly | NX-OS version mismatch — use config from same NX-OS version as switch | 🔧 CHECK |
| Mgmt switch ports not forwarding | Verify: show vlan brief — both ports must show in VLAN 100 | 🔧 CHECK |

# 12.  Final Pre-POAP Checklist

| Task / Component | Details | Status |
| --- | --- | --- |
| Red Hat Server static IP set | ip a — shows 192.168.1.100/24 | ☐ TODO |
| dhcp-server installed | dnf install dhcp-server | ☐ TODO |
| DHCP configured with Opt 66/67 | /etc/dhcp/dhcpd.conf edited with POAP options | ☐ TODO |
| DHCPDARGS set to interface | /etc/sysconfig/dhcpd — DHCPDARGS=eth0 | ☐ TODO |
| dhcpd running | systemctl status dhcpd → active (running) | ☐ TODO |
| tftp-server installed | dnf install tftp-server | ☐ TODO |
| tftp.socket running | systemctl status tftp.socket → active | ☐ TODO |
| Firewall rules added | firewall-cmd --list-all shows dhcp, tftp, ssh | ☐ TODO |
| SELinux TFTP boolean set | setsebool -P tftp_home_dir on | ☐ TODO |
| poap_script.py in /var/lib/tftpboot | ls -la /var/lib/tftpboot/poap_script.py | ☐ TODO |
| switch_config.cfg in tftpboot | ls -la /var/lib/tftpboot/switch_config.cfg | ☐ TODO |
| Files owned by nobody | chown nobody:nobody /var/lib/tftpboot/* | ☐ TODO |
| TFTP self-test passed | tftp 192.168.1.100 → get poap_script.py → success | ☐ TODO |
| Config has 'no boot poap enable' | grep 'boot poap' /var/lib/tftpboot/switch_config.cfg | ☐ TODO |
| Config ends with copy running | tail -5 /var/lib/tftpboot/switch_config.cfg | ☐ TODO |
| Mgmt switch VLAN 100 created | show vlan brief on mgmt switch | ☐ TODO |
| Both ports in VLAN 100 | show interface status on mgmt switch | ☐ TODO |
| Ping server from mgmt switch | ping 192.168.1.100 vrf default — success | ☐ TODO |
| Cables all connected | Server → Mgmt Switch → Nexus 3K mgmt0 | ☐ TODO |
| Switch config erased | write erase done on Nexus 3K | ☐ TODO |